Report of the Final Project Done in complete Fulfilment of the Requirements for the    Award of

One Month Internship completion certification

# SMART ENERGY METER

### FINAL PROJECT REPORT

Done by

### Ishan Jiji George                            Sherry John
Manuel James                             Sona Salu

Under the guidance of

Mrs. Induja & Mrs. Aishwarya

### Students   In

Electronics & Communication Engineering

@
                              Amal Jyothi College of Engineering

JUNE -2025

### INTRODUCTION

In today’s digital era, efficient energy management is crucial for both households and industries. Traditional energy meters require manual readings and provide limited insights into power consumption. To address these limitations, this project introduces a Smart Energy Meter with IoT Billing System, which automates energy monitoring and billing using modern embedded and cloud technologies.The system is built around the ESP8266 NodeMCU, which reads power usage data through a current sensor connected via the ADS1115 ADC module. The collected data is processed in real-time and uploaded to Firebase, enabling users to track their electricity consumption remotely.
To enhance user convenience, the system also calculates the monthly bill based on energy usage and automatically sends it via email using services like SMTP2GO or Google Apps Script. This eliminates the need for manual billing and improves accuracy, transparency, and user engagement. By combining embedded systems with cloud and IoT technologies, this project promotes smart energy usage, supports digital transformation, and lays the foundation for future smart grid integration.

### OBJECTIVE

The objective of this project is to create a Smart Energy Meter that leverages IoT technology to automate the process of energy monitoring and billing. Traditional energy meters require manual readings and offer limited visibility into real-time power usage. This system overcomes those limitations by using an ESP8266 NodeMCU microcontroller and an ADS1115 ADC to accurately measure current consumption. The collected data is uploaded to Firebase, allowing users to remotely access and monitor their energy usage at any time through an internet-connected device.

Additionally, the system calculates the monthly electricity bill based on the total energy consumed and predefined tariff rates. Using email APIs like SMTP2GO or Google Apps Script, it automatically sends the bill to the user’s email address. This approach improves accuracy, eliminates manual effort, promotes energy awareness, and supports digital transformation in energy management. The system is also cost-effective and easily scalable for wider applications.

| COMPONENTS | SPECIFICATION | REQUIREMENT |
| --- | --- | --- |
| ESP8266 NodeMCU | WiFi mod | 1 |
| ZMPT101B Voltage Sensor | Single phase | 1 |
| AD1115 Module |  | 1 |
| SCT013 Transformer |  | 1 |
| BC547 Transistor |  | 1 |
| RESISTOR | 1K,10K | 1,2 |
| CHANNEL RELAY | 5V | 1 |
| CAPACITOR | 0.1uF | 1 |

COMPONENT REQUIRED

### WORKING OF EACH COMPONENT IN THE SMART ENERGY METER

#### ESP8266 Microcontroller (Data Processing & Cloud Connectivity)

- The ESP8266 acts as the brain of the system, reading sensor values and controlling the output devices.

- It connects to WiFi and transmits data to Firebase, enabling remote monitoring.

- Based on sensor data, it activates the LED and buzzer when gas leakage is detected..

### ZMPT101B Voltage Sensor Module

- The ZMPT101B is an AC voltage sensor module used to measure mains voltage accurately and safely. It features a high-precision voltage transformer and operational amplifier circuit to provide an analog output proportional to the input AC voltage. The module is often used alongside a current sensor and ADC to build a complete energy monitoring system. It isolates the high voltage side from the microcontroller, offering protection and stability.

### 3. ADS1115 - 16-bit I2C ADC Module

- The ADS1115 is a precision analog-to-digital converter (ADC) that provides a 16-bit resolution and communicates over the I2C bus. It has four input channels and is often used to interface with analog sensors that produce low-voltage outputs. Its high resolution makes it perfect for accurately measuring signals like those from current or voltage sensors. It also includes a programmable gain amplifier (PGA), allowing amplification of small signals before digitization.

### SCT-013-000 Current Sensor

- The SCT-013-000 is a non-invasive split-core current transformer designed to measure AC current without needing to cut or disconnect the wire. It clips around a live wire and produces a small analog voltage proportional to the current flowing through the wire, which can then be read by a microcontroller. It’s ideal for energy monitoring applications, especially in smart meters and home automation systems. This model can measure currents up to 100A and outputs a voltage that needs to be fed into an ADC for precise measurement.

### BC547 NPN Transistor

- The BC547 is a general-purpose NPN bipolar junction transistor (BJT) used for switching and amplification. It can handle collector currents up to 100 mA and voltages up to 45V. When a small current is applied to its base, it allows a larger current to flow from collector to emitter. It is commonly used in control circuits, relay driving, and signal amplification in analog and digital electronics.

### Resistor (1kΩ)

This through-hole resistor has color bands brown, black, red, and gold, which indicates a resistance of 1,000 ohms (1kΩ) with a tolerance of ±5%. Resistors are passive components that limit current flow and divide voltages in electrical circuits. They are essential for protecting components, controlling biasing conditions in transistors, and setting signal levels in sensors and amplifiers.

### 1-Channel 5V Relay Module

- The 2-channel relay module allows low-power control of high-voltage devices. It operates at 5V and consists of two electromagnetic relays that can switch AC (up to 250V/10A) or DC loads (up to 30V/10A). Each relay can be triggered independently using digital signals from a microcontroller such as Arduino, NodeMCU, or ESP32. It acts as a safety interface to isolate control circuits from high-voltage components, making it ideal for smart home automation and industrial applications.

### 7.    Ceramic Capacitor (Marked 104)

- This is a ceramic disc capacitor commonly used in electronic circuits for decoupling or filtering. The marking “104” represents its capacitance value, where the first two digits are significant figures and the third digit indicates the number of zeros to follow. So, "104" means 10 followed by 4 zeros in picofarads, which is 100,000 pF or 100 nF. These capacitors are widely used for bypassing high-frequency noise to ground, stabilizing voltage, and ensuring smooth operation of microcontrollers and ICs.

#### 8. Firebase Realtime Database (Cloud Logging & Alerts)

- The ESP8266 uploads gas, temperature, and humidity levels to Firebase.

- Users can access this data remotely to ensure home safety.

- Firebase stores alert statuses (Gas Leak Detected or Safe), ensuring easy monitoring.

### WORKING PRINCIPLE

The Smart Energy Meter system is designed to monitor electricity consumption in real time, calculate energy usage in kilowatt-hours (kWh), and automatically generate and email monthly bills using IoT and cloud technologies.

#### 1. Current Measurement

- The electrical current flowing through a load (home or industrial device) is measured using a Current Sensor, such as a CT sensor (e.g., SCT-013-000) or a shunt resistor.

- These sensors produce a small analog voltage signal proportional to the current.

#### 2. Analog-to-Digital Conversion (ADS1115)

- Since the ESP8266's onboard ADC is not accurate enough, the ADS1115 (16-bit ADC module) is used to read the analog signal from the sensor.

- The ADS1115 converts this small analog voltage into a high-precision digital value using I2C communication.

- This allows for highly accurate current measurement, even at low current levels.

#### 3. Power and Energy Calculation (on ESP8266)

- The ESP8266 reads the current value from ADS1115 and multiplies it by a fixed or measured voltage value (typically 230V for AC systems) to calculate instantaneous power (Watts) using:

Power (W)=Voltage (V)×Current (I)\text{Power (W)} = \text{Voltage (V)} \times \text{Current (I)}Power (W)=Voltage (V)×Current (I)

- To calculate energy consumption, the power is integrated over time using:

Energy (kWh)=Power (W)×Time (hr)1000\text{Energy (kWh)} = \frac{\text{Power (W)} \times \text{Time (hr)}}{1000}Energy (kWh)=1000Power (W)×Time (hr)​

- This energy value is continuously accumulated to keep track of total usage.

#### 4. Data Upload to Firebase (Cloud Storage)

- The ESP8266 connects to Wi-Fi and uses the Firebase ESP Client Library to upload:

  - Total energy used (in kWh)

  - Calculated bill amount (e.g., ₹7.5 per kWh)

  - Timestamp of the last reading

- This data is stored under a specific path in Firebase Realtime Database, such as:

bash

CopyEdit

/SmartMeter/energy

/SmartMeter/bill

/SmartMeter/timestamp

- This allows real-time monitoring and remote access via any internet-enabled device.

#### 5. Automated Bill Generation & Email Delivery

- A Google Apps Script runs on a time-based trigger (e.g., monthly) and fetches the data from Firebase.

- It formats the bill (energy used, bill amount, timestamp) into a readable email using HTML.

- The script sends the bill via email to the user using services like:

  - SMTP2GO

  - SendGrid

  - IFTTT

  - or MailApp in Google Apps Script

CIRCUIT DIAGRAM

RESULT  OBTAINED

CODE FOR THE PROGRAM

Google app script code:

function sendEnergyBillEmail() {

const url = 'https://nexus-4-b9919-default-rtdb.firebaseio.com/SmartMeter/SmartMeter.json';

const response = UrlFetchApp.fetch(url);

const data = JSON.parse(response.getContentText());

const energy = data.energy || 0;

const bill = data.bill || 0;

const timestamp = data.timestamp || new Date().toString();

const subject = `⚡ Monthly Bill ⚡ ${energy.toFixed(2)} kWh`;

const body = `

Hi Sona,<br><br>

Here is your Smart Energy Meter usage report:<br><br>

🔌 <b>Energy Used:</b> ${energy.toFixed(2)} kWh<br>

💰 <b>Bill Amount:</b> ₹${bill.toFixed(2)}<br>

🕒 <b>Timestamp:</b> ${timestamp}<br><br>

📟 Smart Meter | Firebase + Google Apps Script

`;

MailApp.sendEmail({

to: "sonasalu27@gmail.com",

subject: subject,

htmlBody: body

});

Arduino code:

#include <Wire.h>

#include <Adafruit_ADS1X15.h>

#include <ESP8266WiFi.h>

#include <Firebase_ESP_Client.h>

#include <NTPClient.h>

#include <WiFiUdp.h>

// Enable serial debugging

#define DEBUG true

// WiFi credentials

#define WIFI_SSID "5.0"

#define WIFI_PASSWORD "ishan002"

// Firebase credentials

#define API_KEY "AIzaSyCn697xTLvtC7Y2tQEvaTrWPwIB6bP3tjE"

#define DATABASE_URL "https://nexus4-39b9c-default-rtdb.firebaseio.com/"

#define USER_EMAIL "sonasalu2026@ec.ajce.in"

#define USER_PASSWORD "MYpassword@1234"

// Firebase objects

FirebaseData fbdo;

FirebaseAuth auth;

FirebaseConfig config;

// ADS1115

Adafruit_ADS1115 ads;

// Time

WiFiUDP ntpUDP;

NTPClient timeClient(ntpUDP, "pool.ntp.org", 19800, 60000); // IST offset

// Tracking

float totalEnergy = 0; // Wh

float billAmount = 0;

unsigned long lastUploadTime = 0;

// Constants

const float pricePerKWh = 7.0;

const float voltageRMS = 230.0;

const float currentScale = 100.0;

const float offsetVoltage = 0.5;

const float mVperStep = 0.1875F;

void setup() {

Serial.begin(115200);

WiFi.begin(WIFI_SSID, WIFI_PASSWORD);

if (DEBUG) Serial.print("Connecting to WiFi");

while (WiFi.status() != WL_CONNECTED) {

delay(500);

if (DEBUG) Serial.print(".");

}

if (DEBUG) Serial.println("\nConnected!");

if (!ads.begin()) {

if (DEBUG) Serial.println("ADS1115 init failed!");

while (1);

}

ads.setGain(GAIN_ONE);

timeClient.begin();

config.api_key = API_KEY;

auth.user.email = USER_EMAIL;

auth.user.password = USER_PASSWORD;

config.database_url = DATABASE_URL;

Firebase.begin(&config, &auth);

Firebase.reconnectWiFi(true);

if (DEBUG) Serial.println("System initialized");

}

String ICACHE_FLASH_ATTR getFormattedDateTime() {

timeClient.update();

time_t epochTime = timeClient.getEpochTime();

struct tm *ti = localtime(&epochTime);

char buffer[25];

sprintf(buffer, "%04d-%02d-%02d %02d:%02d:%02d",

ti->tm_year + 1900, ti->tm_mon + 1, ti->tm_mday,

ti->tm_hour, ti->tm_min, ti->tm_sec);

return String(buffer);

}

void loop() {

int16_t adcValue = ads.readADC_SingleEnded(0);

float voltage = adcValue * mVperStep / 1000.0;

float current = abs((voltage - offsetVoltage) * currentScale);

static float filteredCurrent = 0;

filteredCurrent = 0.9 * filteredCurrent + 0.1 * current;

float power = filteredCurrent * voltageRMS;

float energy = power / 3600.0;

totalEnergy += energy;

billAmount = (totalEnergy / 1000.0) * pricePerKWh;

if (DEBUG) {

Serial.printf("Voltage: %.4f V\n", voltage);

Serial.printf("Current: %.2f A\n", filteredCurrent);

Serial.printf("Power: %.2f W\n", power);

Serial.printf("Energy: %.4f kWh\n", totalEnergy / 1000.0);

Serial.printf("Bill: ₹%.2f\n", billAmount);

}

if (millis() - lastUploadTime >= 10000) {

lastUploadTime = millis();

String path = "/SmartMeter/";

String timestamp = getFormattedDateTime();

FirebaseJson json;

json.set("energy", totalEnergy / 1000.0);

json.set("current", filteredCurrent);

json.set("power", power);

json.set("bill", billAmount);

json.set("timestamp", timestamp);

if (Firebase.RTDB.updateNode(&fbdo, path.c_str(), &json)) {

if (DEBUG) Serial.println("Data uploaded to Firebase");

} else {

if (DEBUG) Serial.println("Firebase upload failed: " + fbdo.errorReason());

}

}

delay(1000);

}

Applications of the Smart Energy Meter

#### 1. Residential Energy Monitoring

- Enables homeowners to monitor their real-time electricity usage.

- Helps reduce electricity bills by identifying energy-hungry appliances.

#### 2. Industrial and Commercial Use

- Tracks energy usage in factories, shops, and offices.

- Helps in load balancing, peak hour usage tracking, and efficient energy management.

#### 3. Smart Apartments and Housing Societies

- Used to bill individual flats based on real consumption.

- Helps apartment management in fair energy distribution and billing.

#### 4. Remote and Rural Electrification Projects

- Enables energy tracking in remote areas where manual reading is difficult.

- Data can be accessed by utility providers from central offices via the internet.

#### 5. Smart Grid Integration

- Forms a building block of smart grids by providing real-time consumption data.

- Assists utility companies in demand forecasting and dynamic pricing.

#### 6. Educational and Research Institutions

- Useful in labs and classrooms to demonstrate IoT-based smart metering.

- Aids in research on energy efficiency, automation, and embedded systems.

#### 7. Co-working Spaces and Rental Units

- Enables billing based on exact energy consumption for different users or tenants.

- Avoids disputes over shared electricity usage.

### ADVANTAGES OF THE METER

  Real-Time Monitoring: 
View live electricity usage remotely via cloud (Firebase).
  Automated Billing:  
Generates and sends monthly bills automatically via email APIs 
  High Measurement Accuracy:  
ADS1115 provides 16-bit precision for reliable current sensing.
  Remote Accessibility:
Access energy data from anywhere using any internet-connected device.
  User-Friendly Operation: 
Simple installation and intuitive system for both residential and industrial users.
  Cost and Time Efficiency: 
Saves manual labor costs and speeds up billing and data collection.

### CONCLUSION

The Smart Energy Meter with IoT Billing System offers a modern solution to traditional energy monitoring by combining real-time data acquisition, cloud storage, and automated billing. Utilizing components like the ESP8266 microcontroller, ADS1115 ADC, and Firebase, this system ensures accurate tracking of energy usage and seamless communication over the internet. By automating the billing process using email APIs, it eliminates manual errors and enhances user convenience. This project not only promotes energy awareness and accountability but also supports the digital transformation of utility services. With potential for scalability, integration with mobile apps, and advanced analytics, the system sets a strong foundation for the future of smart energy management in both residential and industrial sectors. Ultimately, it empowers users to make informed decisions, conserve energy, and reduce operational costs efficiently.